The isotope 40-K makes up 0.0117% of all natural potassium, which is abundant in the earth. The half-life of 40-K is 1.29x109 yr, and it decays by beta emission or electron capture. In electron capture, which occurs 10,7% of the time, an excited state of 40Ar is formed, which in turn emits a 1.46 MeV gamma photon when it drops to the ground state.

Massa di potassio = 0.3861 * massa nitrato di potassio (KNO3)

Massa di potassio = 0.5235 * massa cloruro di potassio (KCl)

KNO3 contain roughtly 38,61 % potassium by weight, so 1 kg of KNO3 contains 386,1g of potassium. This amount of potassium has:

radioactive atoms of 40K

Since the half-life is 1.29x109 yr= 4,07x1016 s, the activity of 1 kg of KNO3 is

The detector absolute efficiency  is 0,0044 (if I use that of a point source) and 11% of decays emit gamma, this give a counting rate of:

| E [keV] | Nuclide | Experimental values |  | DLs: 1.5 + 1.2 + 0.05 mm – FLUKA simulations |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | efficiency | uncertainty | efficiency | uncertainty | Difference % |
| 59,54 | Am-241 | 0,0071 | 0,0003 | 0,0072 | 0,0009 | 1,18% |
| 88,03 | Cd-109 | 0,0259 | 0,0009 | 0,0245 | 0,001 | -5,23% |
| 122,06 | Co-57 | 0,0363 | 0,0012 | 0,0351 | 0,0006 | -3,40% |
| 159,00 | Te-123m | 0,0377 | 0,0012 | 0,0371 | 0,0007 | -1,64% |
| 320,08 | Cr-51 | 0,0244 | 0,0007 | 0,0250 | 0,0007 | 2,62% |
| 391,69 | Sn-113 | 0,0227 | 0,0007 | 0,0213 | 0,0007 | -6,13% |
| 514,01 | Sr-85 | 0,0180 | 0,0005 | 0,0171 | 0,0007 | -5,14% |
| 661,66 | Cs-137 | 0,0155 | 0,0005 | 0,0141 | 0,0011 | -9,15% |
| 898,04 | Y-88 | 0,0113 | 0,0003 | 0,0113 | 0,0015 | 0,15% |
| 1173,24 | Co-60 | 0,0093 | 0,0003 | 0,0093 | 0,0007 | 0,02% |
| 1332,50 | Co-60 | 0,0083 | 0,0003 | 0,0084 | 0,0037 | 1,00% |
| 1836,06 | Y-88 | 0,0065 | 0,0002 | 0,0065 | 0,0063 | 0,15% |

If I use the absolute efficiency fond in leterature for a Marinelli Beacher:

Another literature source get:

from:

Ahmad Saat, Zaini Hamzah “Experimental Determination of the HPGe Spectrometer Efficiency Calibration Curves for Varius Sample Geometry for Gamma Energy from 50 keV to 2000 keV”

Thorium nitrate Activity of TH-232 = (1652)+/- (53) Bq/g

MAESTRO: 2,7 cps